PSIM Demo Version 4.1

(with Motor Drive Module 2.0 and Digital Control Module 1.01)

Copyright (c) 1996-1999 Powersim Technologies Inc.

September 1999

-----------------------------------------------------------------------------------

PSIM is a computer simulation package specially designed for power electronics

and motor control. With fast simulation, friendly user interface and waveform

processing, PSIM provides a powerful simulation environment for power converter

analysis, analog/digital control loop design, and motor drive system studies.

To demonstrate the use and the capability of the software, a number of sample

circuits are provided with the demo version. Sample circuits include power factor

correction, active filter, space vector PWM, VSI with selective harmonic elimination,

multi-level inverter, switchmode power supplies, phase lock loop, linking

with external user-defined C routine through DLL, digital implementation of a PI,

6-pulse and 12-pulse thyristor rectifiers, motor drives, etc.

-----------------------------------------------------------------------------------

1. Simulating a Circuit

=======================

To simulate the sample circuit "chop-1q.sch":

- Start SIMCAD.

- Load the schematic file into SIMCAD.

- Click on the "Run PSIM" icon in the toolbar, or choose "Run PSIM" from the

"Simulate" menu, to start the simulation.

- Click on the "Run SIMVIEW" icon in the toolbar, or choose "Run SIMVIEW" from

the "Simulate" menu, to start SIMVIEW.

- In SIMVIEW, select the curves available for display.

- You may modify the circuit in SIMCAD, run PSIM again, and reload the new

simulation results into SIMVIEW by simply clicking on the "Run SIMVIEW" icon

in SIMCAD.

2. Creating a New Circuit in SIMCAD

===================================

The following shows a step-by-step procedure to create the sample circuit

"chop-1q.sch".

- Start SIMCAD. Click on the icon "New" in the toolbar (or select "New"

from the "File" menu).

- From the "Elements" menu, go to "Sources"/"Voltage" and select "Vdc"

for the dc voltage source. The cursor will change to the image of a

dc voltage source.

- Place the source on the screen by left clicking the mouse. Now the

cursor changes to another dc voltage source image. You can either click

on the "Select" icon in the toolbar to get rid of the voltage source image,

or simply ignore it and proceed to get the second element from the element menu.

- From the "Elements" menu, go to "Power"/"Switches" and select "IGBT".

An upright IGBT image will appear. Right click the mouse to rotate the image.

In this case, you need to click 3 times to get the orientation as required.

- Repeat the same process, get diode ("DIODE") from "Power"/"Switches",

inductor ("L"), capacitor ("C"), and resistor (R) from "Power"/"RLC Branches".

- From "Other"/"Probes", get single-ended voltmeter ("VP"), double-ended

voltmeter ("VP2"), and ammeter ("IP"). "VP" is to measure the node-to-ground

voltage, "VP2" is to measure the voltage between two nodes, and "IP" is to

measure the current through a branch (in this case, the current through

the inductor).

Instead of using the ammeter "IP", you can also view the inductor current

by simply setting the inductor current flag to 1.

- From "Power"/"Switches", get the switch gating block "GATING". There are two

ways a switch is controlled. One is through the gating block. The gating

block output is connected to the gate node of the switch. Refer to the

on-line help (by double clicking on the gating block image and then clicking

on "Help") for its usage.

- From "Other", get the ground element "ground".

Note that for all the elements above, you can also get them by simply clicking

on the corresponding images in the element bar at the bottom of the window.

- Click on the "Wire" icon in the toolbar (or select "Wire" in the "Edit"

menu). The cursor will change to the image of a pen. Left click the mouse and

drag the mouse to draw a wire.

- To create the circuit on the right, you can either build the circuit as before,

or copy the circuit you already built and make modifications. To copy the circuit

on the left, click on the "Select" icon, left click the mouse and drag the mouse

to define the region. After the region of the circuit is selected, a double line

frame will appear. Click on the "Copy", and then "Paste" icon.

- The circuit on the right uses the second way of controlling a switch, that is,

with a switch controller. The input of the switch controller comes from a control

circuit. The on-off switch controller used in this circuit can be obtained by

selecting "ONCTRL" from "Other"/"Switch Controllers".

The on-off switch controller behaviors like a switch base driver circuit. Unlike the

real base driver circuit, however, a value of 0 will turn the transistor off, and a

value of 1 will turn the switch on.

The gating signal can be obtained by comparing a dc voltage source with a

triangular voltage source. The comparator can be obtained by selecting

"COMP" from "Control", and the triangular voltage source "VTRI" from

"Sources"/"Voltage"

- Once the connection is complete, double click on the image of each element to set

the parameters. For example, to set the inductor parameters, double click on

the inductor image and type in the values.

- To set the simulation control parameters, click on "Simulation Control" from

the "Simulate" menu and an image of a clock will appear. By double clicking

on the clock image, you can specify the simulation time step, total study

time, etc.

Once the circuit is complete, you are ready to run the simulation. Click on

the "Run PSIM" icon (or go to the "Simulate" menu and select "Run PSIM") to run PSIM.

Once the simulation is complete, PSIM asks your confirmation to exit. If you wish to

exit PSIM automatically, select "Auto-exit PSIM" in the "Option" menu. To view the

simulation waveforms, click on the "Run SIMVIEW" icon (or go to the "Simulate" menu

and select "Run SIMVIEW").

You can go back to SIMCAD to modify the circuit or parameters. Run PSIM

again, and click on the "Run SIMVIEW" icon in SIMCAD to see the new waveforms. The

old waveforms will be updated automatically. Alternately, you can also click on the

"Re-load data" icon in SIMVIEW to load the new simulation results.

-----------------------------------------------------------------------------------